| 名称：全自动高速铣槽机 型号：ICM-6000 用途：用于在ISO7816标准卡上铣槽 ，特别适合双界面卡 片高精度要求铣槽。 优势：1、铣槽精度高，特别适合对精度要求较高的槽位的生 产，如双界面卡铣槽。 2、产品电路符合CE标准，操作安全可靠 3、每个锣头可单独通过软件界面编辑多达5层不同规 格的铣槽，且两个锣头可以铣不同的图形。 4、每层铣槽走刀路径，起点，终点位置用户可以自己 编辑，使用户更加方便控制铣槽质量及铣槽速度。 5、软件密码级别设置，不同级别密码可设置不同参数 管理 权限，如操作员密码，管理员密码 |  |
| --- | --- |
| 功能及特性： 1、ICM-6000全自动高精度铣槽机采用研华IPC加安川高精度伺服系统控制。 2、双卡匣设计，一次送两张卡到传送皮带。 3、铣槽前卡片方向检测，放置反的卡片自动剔除。 4、传送皮带采用卡片纵向送料,使机器结构更紧凑,传动快捷准确。 5、采用机械方式定位，定位精度远高于传统气缸定位，夹具采用四组进口轴承导向，铣槽时保证卡片压紧。 6、采用高速水冷锣头,确保高速运转精度,延长刀具及锣头使用寿命。 7、双锣头铣槽设计,一次同时完成两张卡铣槽, 加工效率高。 8、修改槽的尺寸可直接在专用的生产软件界面编制图形输入完成，设定时图形示意图即时呈现,操作方便直观。 9、具有铣槽时及铣槽后的两次气动全封闭吸尘清洁装置，确保吸尘干净。 10、铣槽后装有电性能检测工位检测双界面卡线圈通断，不良卡会被排到尾部废卡盒。 11、设备左右急停按钮,方便操作;设备有良好接地,安全可靠。 12、设备装有安全门禁开关,运行中开门将自动停机,确保操作安全。 |  |
| 技术规格： 类 别; 内容描述; 类 别; 内容描述. 最大铣槽尺寸; 14X14mm ( 可按客户要求配置); 适用物料; PVC, ABS, PETG，PC，光面卡，透明料的 ISO7816标准卡. 铣槽精度; X,Y (位置) = +/- 0.05mm Z(深度) = +/- 0.015mm; 锣头转速; 1000-25000 转/分钟. 产 能; 5000-6000 张卡/小时; 产品合格率; 99.0％. 耗气量; 400L/min; 电 源; AC380V, 50/60Hz, 3kw（不包括吸尘机）. 重 量; 750 kg; 气 压; 6kg/cm2，无水与无油。. 外形尺寸; L2000mm×W800mm×H2000mm; 使用环境; 温度 22℃，+ 3℃，干燥和无尘. |

声明：以上参数如变更恕不另行通知。

| Automatic High Precision IC Card Milling Machine Model: ICM-6000 Purpose: For ISO7816 standard card cavity milling, particularly for dual interface cards which requires high-precision cavity milling. Advantages: - High accuracy of cavity milling, particularly suitable for high-precision required production, like cavity milling on dual interface card. - Circuit meets CE requirements. Machine operation is safe and reliable. - Each spindle can accomplish different cavities with 5 layers. Two spindles deal with different graphics. - User can edit the milling route, start point, end point of each layer, which is convenient to control the speed and quality of cavity milling. - The software password has grades. Different grades have different parameter management rights, such as operator password, administrator password. |  |
| --- | --- |
| Features: - Adopting IPC and Yaskawa high precision servo control system - Double-magazines design, two cards feeding at the same time - Card orientation detection before cavity milling which can reject anti-card automatically. - Transfer belt controlled by servo can feeding automatically, quickly and accurately. - Mechanical position, whose position accuracy is far higher than that positioned by cylinder. The fixture adopts four sets imported bearings to guide. Cards can be compressed when milling. - Adopting high-speed, water cooling spindle which ensures high-speed running precision and make bits & spindle longer life time. Dual-spindle design, can mill two cards at one time, processing efficiency is high. - Cavity dimension adjusted by the specialized production software interface and the sketch shown instantly. The operation is very easy. - Twice pneumatic enclosed dust vacuum cleaning device on & after milling can ensure dust removed. - Electrical performance testing to verify the conduction of antenna on dual interface card. Unqualified cards to be rejected into the waste magazine. - Emergency stop buttons on both sides. Well grounding. - Safe access control switch. Automatic outage once the door opens. |  |
| Specifications: Item; Details; Item; Details. Max. milling dimension; 14X14mm (configured by clients’ requirements); Suitable material; PVC, ABS, PETG, PC, smooth card, ISO7816 standard card with transparent film. Milling accuracy; X,Y (position) = +/- 0.05mm Z(depth) = +/- 0.015mm; Spindle rotate speed; 1000-25000 turns/min. Throughput; 6000 cards/hr (depends on size of cavity); Yield; 99.0%. Air consumption; 400L/min; Power; AC380V, 50/60Hz, 3KW (excluding vacuum). Weight; 750 kg; Air pressure; 6kg/cm2, clean, dry and oil-free.. Dimension; L2000mm×W800mm×H2000mm; Environment; Constant temperature 22℃, ±3℃, dry and dust free.. |

Notes: above specification is subject to change without prior notice due to continuous improvement.