| 名称：全自动IC卡封装机 型号：ICI-6000 用途：将条带模块备胶、冲切并封装在已铣槽的卡基上。 优势：1、PLC程序自动控制，中英文触摸屏显示及操作; 2、集备胶、冲切、封装及检测于一体。 |  |
| --- | --- |
| 功能及特性： 1、双卡匣自动上下料。 2、自动检测槽位并进行封装。 3、先点焊定位，然后进行热焊A、B二站，最后进行冷压。 4、封装后自动检测，并自动将废卡剔除。 5、自动检测模块，并对坏IC模块进行识别并打孔坏。 6、针对不同模块，可灵活更换备胶、冲切模具。 |  |
| 技术规格： 类 别; 内容描述; 类 别; 内容描述. 最高运行速度; 6000UPH; 适用物料; PVC, ABS, PET-G材料的ISO7816 标准卡. 铣槽驱动精度; X, Y = +/- 0.015mm; 封装温度; 100℃ ～300℃可调节. 封装压力; 3 ～ 20Kg/cm2; 重 量; 2500kg. 电源/功率; AC220V，50/60Hz，1.5KW; 外形尺寸; (L)1960mm x (W)800mm x (H)2050mm. 耗气量; 250L/min; 使用环境; 温度 22℃，+ 3℃，干燥和无尘. |

声明：以上参数如变更恕不另行通知。

| Automatic IC Card Implanting Machine Mode：ICI-6000 Purposes: The machine is to laminate module tape, punch module from the tape and implant module onto milled plastic cards. Advantages: - PLC program control. Chinese/English touch screen and operation. - Integration of module tape gluing, module punching, module implanting and testing. |  |
| --- | --- |
| Features: - Double card magazines to load/unload cards. - Automatically check card cavity and implant modules. - Spot weld for positioning, hot weld stations A & B, then cold press. - Automatically check card after implanting and reject defective cards. - Identify cards with bad module and punch hole on cards. - For different module, flexible replacement of tooling of module tape gluing and module punching. |  |
| Specifications: Item; Details; Item; Details. Max. Speed; 6000UPH; Suitable material; ISO7816 standard card of PVC, ABS, PET-G. Milling accuracy; X, Y = ± 0.015mm; Temperature; 100℃ ～300℃ (adjustable). Implanting pressure; 3 ~ 20Kg/cm2; Weight; 2500kg. Power; AC220V, 50/60Hz, 1.5KW; Dimension; (L)1960mm x (W)800mm x (H)2050mm. Air consumption; 250L/min; Environment; Constant temperature 22℃, ±3℃, dry and dust free.. |

Notes: above specification is subject to change without prior notice due to continuous improvement.